PRIMITIVE VIDEO PLAYER DRIVER SUPPORT

Paradise Software, Inc.

55 Princeton-Hightstown Road

Suite 109

Princeton Junction, NJ 08550

(609) 275-4475

OVERVIEW

The following documents the a programmers interface to the device control

library used by VAA and DAND.  The interface is in the form of a API; a

device object is created within the C application which represents a

video source controlled by the Sun Workstation.  A standard set of actions

can be invoked on a video source to control the video player.

An Example program:

#include <X11/Xlib.h>

#include <xview/xview.h>

#include "video.h"

#include "player.h"

static VideoIn video;			/* Pointer for video input object		*/

static Frame   VAAFrame;		/* Pointer for window frame on display	*/

static Player *player;			/* Pointer to video source object		*/

/* Quit Notify routine */

Notify_value quit(client,status)

Notify_client client;

Destroy_status status;

{

if (status == DESTROY_CLEANUP) {

xv_destroy(video);

close(player->fd);

return(notify_next_destroy_func(client,status));

}

return(NOTIFY_DONE);

}

/* Sample code to display a video frame and control the video source */

main(argc, argv)

int argc;

char *argv[];

{

/* Init XView environment */

xv_init(XV_INIT_ARGC_PTR_ARGV, &argc, argv, NULL);

/* Create window frame for video input canvas */

VAAFrame = (Frame) xv_create(NULL,FRAME,NULL);

/* Notice window quit to destroy the video canvas */

notify_interpose_destroy_func(frame,quit);

/* Create a simple video frame with Live Video */

video = (VideoIn) xv_create(frame,VIDEO_IN,

VIDEO_LIVE,			TRUE,

VIDEO_SQUEEZE,		TRUE,

NULL);

if (video == NULL) {

printf("Problem creating video canvas -- possible sync problem ---\n");

printf("check input connection\n");

exit(1);

}

window_fit(VAAFrame);

/* Create a player control device, in this example we have a Pioneer LD-V4200

connected to serial port ttyb, and configured at 9600 baud */

player = (Player *) InitializeVideo("/dev/ttyb",PIONEER_MODEL,9600);

if (Player == NULL) {

printf("Couldn't initialize video player\n");

exit(1);

}

/* Display the frames 450 thru 2000 */

ShowDiscSegment(player,450,2000);

xv_main_loop(frame);

}

INTERFACE FUNCTIONS

*** The interface is preliminary and will change in future releases.

*** Use at your own risk.

The include file player.h contains the external definitions necessary

to use the library.  Note that not all operations are possbile on all

supported video players, exceptions are noted were they exist.

The operations supported are:

#######################################################################

Player *InitializeVideo(device,model,speed)

#######################################################################

Called to initialize the connection to the video player.  Arguments are:

device : char * pointer to the name of the serial device connected to the

video player.  Usually one of "/dev/ttya" or "/dev/ttyb".

model  : One of the model types supported -

PIONEER_MODEL => LD-V4200

SONY_MODEL    => 1500,1550,2000

POD_MODEL     => TQ-3031F

NEC_MODEL	  => PC-VCR

LIVE_MODEL	  => Any uncontrollable source

FILE_MODEL	  => JPEG compressed video file (unsupported)

speed  : Speed supported by video player, one of 1200,2400,4800,9600.

#######################################################################

int PlayerStatus(player)

#######################################################################

Returns a status code from the player to indicate the internal state.

Possible values are :

PLAYER_STATUS_UNKNOWN

PLAYER_STATUS_STOP

PLAYER_STATUS_STILL

PLAYER_STATUS_PLAY

PLAYER_STATUS_EMPTY

#######################################################################

int PlayerFrameNumber(player);

#######################################################################

Returns the current frame number as an int.  Only valid on CAV style

laserdiscs for Sony and Pioneer disc players.

#######################################################################

int ShowDiscFrame(player,frame);

#######################################################################

Seek to the requested frame and display as a still frame.  Only valid

on CAV style laserdiscs for Sony and Pioneer disc players.

#######################################################################

int ShowDiscSegment(player,start,end);

#######################################################################

Seek to "start" frame and player video at normal speed until the "end"

frame.  Only valid on CAV style laserdiscs for Sony and Pioneer disc

players.

#######################################################################

int PlayerPlay(player);

#######################################################################

Begin playing video in the forward direction at the current frame.

#######################################################################

int PlayerReverse(player);

#######################################################################

Begin playing video in the reverse direction at the current frame.

#######################################################################

int PlayerStop(player);

#######################################################################

Stop any video playback and spin down the video source (tape or disc)

#######################################################################

int PlayerContinue(player);

#######################################################################

Continue operation interrupted by PlayerStill.

#######################################################################

int PlayerStill(player);

#######################################################################

Pause the video playback.

#######################################################################

int PlayerStepForward(player);

#######################################################################

Advance the video source one frame in the forward direction.  Only

valid on CAV style laserdiscs for Sony and Pioneer disc players.

#######################################################################

int PlayerStepReverse(player);

#######################################################################

Advance the video source one frame in the backward direction.  Only

valid on CAV style laserdiscs for Sony and Pioneer disc players. Not

supported by the NEC PC-VCR.

#######################################################################

int PlayerSlowForward(player);

#######################################################################

Advance the video source slowly (1/5 - 1/10 normal speed) in the

forward direction.

#######################################################################

int PlayerSlowReverse(player);

#######################################################################

Advance the video source slowly (1/5 - 1/10 normal speed) in the

backward direction.  Not supported by the NEC PC-VCR.

#######################################################################

int PlayerFastForward(player);

#######################################################################

Advance the video source fast (2x - 5x normal speed) in the forward

direction.

#######################################################################

int PlayerFastReverse(player);

#######################################################################

Advance the video source fast (2x - 5x normal speed) in the backward

direction.

#######################################################################

int PlayerIndex(player);

#######################################################################

Causes the video player to display on the video output stream information

relating to the current frame location and perhaps status information.

#######################################################################

int PlayerEject(player);

#######################################################################

Causes the current disk or tape to be ejected from the player.