.ds RH March 14, 1992

.ds CF Confidential Information \(em Do Not Distribute

.TL

PARALLAX JPEG MOVIE INTERFACE

.AU

Paradise Software, Inc.

55 Princeton-Hightstown Road

Suite 109

Princeton Jct. N.J. 08550

(609) 275-4475

.sp

.NH

OVERVIEW

.LP

The following is a description of a software library offering

a high level interface to the creation and playback of JPEG

movie files.  This interface is based on the Parallax Graphics

XView API for the Parallax XVideo-24 compression hardware/drivers.

Currently, this extension is somewhat limited in flexibility,

but should grow to become more complete over time.

.LP

The JPEG movie extension has attempted to create a reasonably

abstract interface to functionality provided by the XVideo-24

board and the XView interface supported by Parallax Graphics.

Application software written for the extension should be able

to move transparently from one JPEG implementation to another.

This JPEG Movie API is designed to handle only the XVideo-24

product with the optional CCube JPEG chip from Parallax Graphics.

.NH

DESCRIPTION

.LP

The JPEG movie extensions were designed for the storage of JPEG

image data (from 1 to N frames) with or without synchronized

audio data.  A set of high level functions have been provided

for the creation and retrival of information stored in this

format.  The functions are divided into two main classes,

dealing with either single frame movies with no audio, or

multi-frame movies with or without audio.

.LP

Below is a sample of a XView program which creates a window on the

display and captures a single still compressed frame for storage

on the hard disk.

.nf

.sp 2

#include <X11/Xlib.h>

#include <xview/xview.h>

#include "Video.h"

#include "jpeg.h"

static Frame frame;

static VideoIn video;

Notify_value quit(client,status)

Notify_client client;

Destroy_status status;

{

JPEGImage *j;

if (status == DESTROY_CLEANUP) {

j = (JPEGImage *) xv_get(video,VIDEO_JPEG_IMAGE);

jpeg_save_still(xv_get(frame,XV_DISPLAY),j,"image");

return(notify_next_destroy_func(client,status));

}

return(NOTIFY_DONE);

}

main(argc, argv)

int argc;

char *argv[];

{

xv_init(XV_INIT_ARGC_PTR_ARGV, &argc, argv, NULL);

frame = (Frame) xv_create(NULL,FRAME,

XV_WIDTH,				320,

XV_HEIGHT,				240,

NULL);

notify_interpose_destroy_func(frame,quit);

video = (VideoIn) xv_create(frame,VIDEO_IN,

VIDEO_LIVE,					FALSE,

VIDEO_Q_FACTOR,				100,

NULL);

window_fit(video);

window_fit(frame);

xv_main_loop(frame);

}

.sp 2

.fi

.LP

In this simple case, a base frame is created with xv_create.

Then a video canvas, identified by the class name VIDEO_IN, is

created as a subwindow of the frame.  Once the video canvas is

created, a single video frame is displayed continuously.  When

the user exits the program, the image is compressed and saved

as an JPEG file.

.NH

MOVIE FILE STRUCTURE

.LP

A JPEG movie file is structured as a linear stream of bytes. The

overall structure of the file looks like this:

.sp

.sp

.fp 1 CO

.DS

|        |       |       |     |       |         |

| fixed  | frame | frame |     | frame |  frame  |

| length |   1   |   2   | ... |   N   | offsets |

| header |       |       |     |       |  table  |

|        |       |       |     |       |         |

.DE

.fp 1 R

.sp

.sp

Within each frame is a linear sequence of instructions and their

operands.  The instructions are encoded within a single byte and

are defined in movie.h  The operands are instruction specific and

of variable length.

.LP

Each file starts with a fixed length record which describes

the attributes of the movie.  This record is defined as follows.

.sp

.sp

\s-1

.nf

typedef struct {

char magic[8];			/* defined as "j_movie"						*/

int version;				/* Version number							*/

int fps;				/* frames per second						*/

int frames;				/* Number of frames in the sequence				*/

int width;				/* Width of each frame 						*/

int height;				/* Height of each frame 						*/

int bandwidth;			/* Bandwidth required for playback				*/

int qfactor;				/* Quanization factor used in compression			*/

int mapsize;				/* Number of colors used in colormap				*/

int indexbuf;				/* Offset in file of frame indexes					*/

int tracks;				/* Number of audio tracks used					*/

int volbase;				/* Baseline for volume control					*/

int audioslice;			/* Number of audio bytes per frame				*/

Audio_hdr audio;			/* Audio header from input file					*/

char filler[FILLER];			/* Pad out this record to 128 bytes				*/

} jpheader;

.fi

\s+1

.LP

Following the fixed data is a sequence of movie frames. A frame is composed

of a set of operations to be performed on playback.  Currently, the only

set of operations defined for JPEG movies are

.sp

.RS

.LP

(1) Load JPEG still

.LP

(2) Load audio track

.LP

(3) End current frame

.RE

.sp

.LP

The EOCF (end of current frame) operation is required for every frame.

.LP

At the end of the JPEG movie is a array of frame offsets.  The record

field \fIindexbuf\fR is the \fIoffset\fR in bytes from the beginning of

the file to an array of ints. \fIoffset+0\fR is the address of the first

frame of the movie, \fIoffset+1\fR is the address of the second frame

and so forth till the last frame.

.NH

ERRORS

.LP

Error messages are produced through the functions defined in

alert.c and can be used as is, or can be modified by the

user and used in place of the standard functions. Each library

functions returns -1 if an error occured, else returns 0 on

success.

.NH

Application Program Interface (API)

.LP

The JPEG movie API currently offers user 10 functions which are

compatibile with the XView XVideo extension.  These functions

are defined below.  Note that all the functions expect as their

first argument a pointer to the X11 DISPLAY structure.

.sp

.nf

int jpeg_free_image(dpy,image)

Display   *dpy;

JPEGImage *image;

.fi

.sp

.RS

.LP

This function is used to destroy a JPEGImage data structure

gotten from a VideoIn object.  It should only be used when

the attribute VIDEO_USE_SHM is not set to TRUE on the VideoIn

object.

.RE

.sp

.nf

int jpeg_save_still(dpy,image,filename)

Display   *dpy;

JPEGImage *image;

char      *filename;

.fi

.sp

.RS

.LP

This function is used to store a single JPEG compressed

image into a UNIX file.  \fIImage\fR is a pointer to a

JPEGImage structure created as a result of accessing the

VIDEO_JPEG_IMAGE attribute.  \fIFilename\fR is a NULL

terminated string which contains the name of the file

in which to store the data.

.RE

.sp

.nf

JPEGImage *jpeg_load_still(dpy,filename)

Display   *dpy;

char      *filename;

.fi

.sp

.RS

.LP

This function is used to retrieve a single JPEG compressed

image from a UNIX file.  \fIFilename\fR is a NULL

terminated string which contains the name of the file

from which to get the image.

.RE

.sp

.nf

int jpeg_save_jfif(dpy,image,filename,qtable)

Display   *dpy;

JPEGImage *image;

char      *filename;

u_char    *table;

.fi

.sp

.RS

.LP

This function is used to store a single JPEG compressed

image into a UNIX file in JFIF format.  \fIImage\fR is

a pointer to a JPEGImage structure created as a result

of accessing the VIDEO_JPEG_IMAGE attribute.  \fIFilename\fR

is a NULL terminated string which contains the name of

the file in which to store the data. \fIQtable\fR is a

pointer to an array of unsigned chars which contains the

luminance and chrominance quantization tables in the

format defined by \fBqtab.h\fR.

.RE

.sp

.nf

JPEGImage *jpeg_load_jfif(dpy,filename)

Display   *dpy;

char      *filename;

.fi

.sp

.RS

.LP

This function is used to retrieve a single JPEG compressed

image from a UNIX file which is in the JFIF format.

\fIFilename\fR is a NULL terminated string which contains

the name of the file from which to get the image. Note

that only the 4:2:2 YUV format is supported for image

display.

.RE

.sp

.nf

int jpeg_open_movie(dpy,filename,fps,qfactor,audio)

Display   *dpy;

char      *filename;

int        fps;

int		   qfactor;

Audio_hdr *audio;

.fi

.sp

.RS

.LP

This function is used to initialize an empty JPEG movie

file. \fIFilename\fR is a NULL terminated string which

contains the name of the file in which to store the data.

\fIFps\fR is an int which specifies the frame speed at

which to play the movie data back at.  It should be a

number between 1 and 30. \fIQfactor\fR specifies the

XVideo Q factor used to control the compression quality

for the movie file.  \fIAudio\fR is a pointer to

a Audio_hdr structure, see \fBAudio_hdr (3)\fR for a

description of its usage.  If \fIaudio\fR is NULL, then

audio usage in this JPEG movie will be ignored.

.RE

.sp

.nf

int jpeg_add_still(dpy,image)

Display   *dpy;

JPEGImage *image;

.fi

.sp

.RS

.LP

This function is used to add the next JPEG compressed frame

to the current movie. \fIImage\fR is a pointer to a

JPEGImage structure.

.RE

.sp

.nf

int jpeg_get_slice(dpy)

Display   *dpy;

.fi

.sp

.RS

.LP

This function returns the number of audio bytes required

to be passed to jpeg_add_audio to insure that the audio

is synchronized with the video on playback.

.RE

.sp

.nf

int jpeg_add_audio(dpy,audio,track)

Display *dpy;

u_char  *audio;

int     track;

.fi

.sp

.RS

.LP

This function is used to add a piece of the audio track

to the movie file. \fIAudio\fR is a pointer to a buffer

which contains at least \fBjpeg_get_slice()\fR bytes of

audio data.  \fITrack\fR is the index of the audio track

under which the data will be accessed.  Currently, this

value should always be 0.

.RE

.sp

.nf

int jpeg_end_frame(dpy)

Display   *dpy;

.fi

.sp

.RS

.LP

This function is used to signal the end of the current JPEG

movie frame and implicitly open up a new frame.

.RE

.sp

.nf

int jpeg_stop_movie(dpy)

Display   *dpy;

.fi

.sp

.RS

.LP

This function is used to signal the end of the JPEG movie

and causes the current movie file to be terminated.

.RE

.sp

.nf

int jpeg_play_movie(dpy,video,filename)

Display *dpy;

VideoIn  video;

char    *filename;

.fi

.sp

.RS

.LP

This function can be used to playback a previously stored

JPEG movie file.  \fIVideo\fR is the VideoIn object into

which the JPEG movie will be rendered. \fIFilename\fR is

a NULL terminated string which contains the name of the

JPEG movie to play.

.RE

.sp

.nf

jpheader *jpeg_movie_header(dpy,filename)

Display *dpy;

char    *filename;

.fi

.sp

.RS

.LP

This function is used to get access to the header information

of a JPEG movie file. The header format is defined in movie.h.

.RE

.sp

.nf

int jpeg_interrupt_movie(dpy)

Display *dpy;

.fi

.sp

.RS

.LP

This function is used to interrupt the playback of a JPEG

movie.

.RE

.NH

Assumptions

.LP

The JPEG movie API assumes the following to be true of

the environment in which it is used:

.RS

.LP

Using SUNOS 4.1 or greater.

.LP

The following options are incorporated into the SUNOS

kernal: IPCSHMEM, LWP, ASYNCIO.

.RE