.ds RH October 31, 1991

.ds CF Confidential Information \(em Do Not Distribute

.TL

XVIEW EXTENSION: PARALLAX VIDEO JPEG INTERFACE

.AU

Paradise Software, Inc.

55 Princeton-Hightstown Road

Suite 109

Princeton Jct. N.J. 08550

(609) 275-4475

.sp

.NH

OVERVIEW

.LP

The following is a description of the JPEG extensions to the

XView API for the Parallax XVideo-24 compression hardware/drivers.

Currently, this extension is extremely limited in flexibility,

but should grow to become more complete over time.

.LP

The XView JPEG extension has attempted to create a reasonably

abstract interface to functionality provided by the XVideo-24

board and the XLIB interface supported by Parallax Graphics.

Application software written for the extension should be able

to move transparently from one JPEG implementation to another.

This XView extension is designed to handle only the XVideo-24

product with the optional CCube JPEG chip from Parallax Graphics.

.NH

DESCRIPTION

.LP

Two new attributes have been added to the VIDEO_IN XView class.

VIDEO_IN is a subclass of the standard XView canvas object and

contains live video superimposed on the canvas paint window.

This video object shares all the normal properties and interactions

of the standard XView classes, including object creation, attribute

modification, and destruction.

.LP

Below is a sample of a XView program which creates a window on the

display and captures a single still compressed frame for storage

on the hard disk.

.nf

.sp 2

#include <X11/Xlib.h>

#include <xview/xview.h>

#include "Video.h"

static Frame frame;

static VideoIn video;

Notify_value quit(client,status)

Notify_client client;

Destroy_status status;

{

JPEGImage *j;

if (status == DESTROY_CLEANUP) {

j = (JPEGImage *) xv_get(video,VIDEO_JPEG_IMAGE);

.

Write data out in some user specific format

.

return(notify_next_destroy_func(client,status));

}

return(NOTIFY_DONE);

}

main(argc, argv)

int argc;

char *argv[];

{

xv_init(XV_INIT_ARGC_PTR_ARGV, &argc, argv, NULL);

frame = (Frame) xv_create(NULL,FRAME,

XV_WIDTH,				322,

XV_HEIGHT,				242,

NULL);

notify_interpose_destroy_func(frame,quit);

video = (VideoIn) xv_create(frame,VIDEO_IN,

VIDEO_LIVE,					FALSE,

VIDEO_Q_FACTOR,				75,

NULL);

window_fit(video);

window_fit(frame);

xv_main_loop(frame);

}

.sp 2

.fi

.LP

In this simple case, a base frame is created with xv_create.

Then a video canvas, identified by the class name VIDEO_IN, is

created as a subwindow of the frame.  Once the video canvas is

created, a single video frame is displayed continuously.  When

the user exits the program, the image is compressed and saved

as an JPEG file.

.NH

ERRORS

.LP

Currently, no JPEG specific error messages are generated.

.NH

ATTRIBUTES

.LP

The class VIDEO_IN offers 6 user controllable attributes which

access the JPEG functionality of the hardware.  These attributes

are defined below.

.NH 2

\fBVIDEO_JPEG_ZOOM\fR

.LP

Setting this attribute to TRUE causes the the video still being

captured to be scaled up to the size of the full video image

width/height constraints before being passed to the JPEG

compression hardware.  This is useful in that the image can be

small in the source window, but can be zoomed up to full size

in the destination window.  The default value is FALSE.

.NH 2

\fBVIDEO_USE_SHM\fR

.LP

Setting this attribute to TRUE causes (if possible) the image

data to be placed in a shared memory structure that is used

by both the server and your application.  This can only be

used if the X server is local to your machine.  This is a

performance enhancement which prevents extra data copying.

The default value is FALSE.

.NH 2

\fBVIDEO_JPEG_ONLY\fR

.LP

Setting this attribute to TRUE causes this canvas to be

specialized for receiving JPEG compressed data only.  The

normal behavior of a video window being connected to a

video stream off the board is disabled, and the object

expects to be passed data via the VIDEO_JPEG_IMAGE

attribute only.  Note that trying to get data from this

object will always fail.  If VIDEO_SQUEEZE is TRUE, then

the image will be scaled to fit into the canvas if it

is larger, else the image will be cropped to fit the

size of the current canvas paint window.  If the image

is smaller than the canvas, then it will be centered

in the canvas.  The default value of this attribute

is FALSE, and it can only be set at creation time. Any

number of VIDEO_JPEG_ONLY canvases can be created on the

screen.

.NH 2

\fBVIDEO_Q_FACTOR\fR

.LP

Setting this attribute controls the the amount of JPEG value

quantization used when compressing the image.  This directly

controls the compression ratios achieved by the CCube chip.

Nominal values are between 50 and 150.  The default value is

100. xv_get can be used to retrieve its current value.

.NH 2

\fBVIDEO_JPEG_IMAGE\fR

.LP

This attribute can be used either to create a JPEG compressed

version of the current video frame, or to load a previously

JPEG compressed video frame.  \fBVIDEO_LIVE\fR must be set

to FALSE before this attribute can be used.  Retrieving the

value of this attribute returns a pointer to a JPEGImage data

structure which contains the relevant data. A JPEGImage

structure is defined as:

.nf

.sp 1

typedef struct {

int   width;

int   height;

int   size;

char *data;

XShmSegmentInfo *info;

} JPEGImage;

.sp 1

.fi

where \fIwidth\fR is the width of the uncompressed frame, \fIheight\fR

is the height of the uncompressed frame, \fIsize\fR is the number of

bytes in the compressed frame, and \fIdata\fR is a pointer to the

compressed data.  \fIinfo\fR is currently unused.  To accessed a

compressed image, one could do:

.nf

.sp 1

j = (JPEGImage *) xv_get(video,VIDEO_JPEG_IMAGE);

.sp 1

.fi

while to uncompress a JPEG image, one would do:

.nf

.sp 1

xv_set(video,VIDEO_JPEG_IMAGE,j,NULL);

.sp 1

.fi

.NH 2

fBVIDEO_EXPOSED\fR

.LP

This is a read-only attribute which will be TRUE once the video

window has been mapped and painted on the screen.  This flag

exists as JPEG compression/decompression operations can only

be applied to a window that has been mapped onto the screen.