.ds RH April 7, 1991

.ds CF Confidential Information \(em Do Not Distribute

.TL

XVIEW EXTENSION: PARALLAX VIDEO INTERFACE

.AU

Paradise Software, Inc.

RD 1 Box 467A

Lambertville, NJ 08530

(609) 397-4142

.sp

.NH

OVERVIEW

.LP

The following is a description of release 1.0 of the XView video

extensions developed by Paradise Software for the Parallax Graphics

family of video hardware products.  It was developed using the

toolkit extension facility supported by XView which allows for

the addition of new objects to the existing class library. These

new objects, or widgets as defined by X11 nomenclature, are in

turn based on the Parallax video extensions to the XLIB toolkit.

.LP

The XView video extension has attempted to create a reasonably

abstract interface to the family of Parallax Graphics video

display boards.  Application software written for the extension

should move transparently from one board to another.  Currently,

this XView extension is designed to handle the VIPER, VideoView

and XVideo boards from Parallax Graphics.

.NH

DESCRIPTION

.LP

Two new object classes have been added to the XView inheritance

hierarchy, VIDEO_IN and VIDEO_OUT.  VIDEO_IN is a subclass of the

standard XView canvas object and contains live video superimposed

on the canvas paint window.  VIDEO_OUT is a subclass of the

standard XView generic object and contains parameters for defining

the region of the screen for display through the video output

capabilities of the hardware. These two new objects share all the

normal properties and interactions of the standard XView classes,

including object creation, attribute modification, and destruction.

.LP

Below is a sample of a XView program which creates a window on the

display and shows live video until the user selects quit from the

frame menu.

.nf

.sp 2

#include <X11/Xlib.h>

#include <xview/xview.h>

#include "video.h"

static VideoIn video;

static Frame frame;

Notify_value quit(client,status)

Notify_client client;

Destroy_status status;

{

if (status == DESTROY_CLEANUP) {

xv_destroy(video);

return(notify_next_destroy_func(client,status));

}

return(NOTIFY_DONE);

}

main(argc, argv)

int argc;

char *argv[];

{

xv_init(XV_INIT_ARGC_PTR_ARGV, &argc, argv, NULL);

frame = (Frame) xv_create(NULL,FRAME,NULL);

notify_interpose_destroy_func(frame,quit);

video = (VideoIn) xv_create(frame,VIDEO_IN,NULL);

xv_main_loop(frame);

}

.sp 2

.fi

.LP

In this simple case, a base frame is created with .itxv_create.

Then a video canvas, identified by the class name VIDEO_IN, is

created as a subwindow of the frame.  Once the video canvas is

created, the video is displayed continously.  If the user moves

or reshapes the window, the video adjusts accordingly.

.NH

ERRORS

.LP

The intent of the XView interface is to make the configuration

of the video hardware as transparent to the developer as possible.

However, some operations require the return of a status code such

that the application writer can determine that his/her requested

operation did not occur as desired.  The XView video extensions

makes use of two procedures for indicating errors to the application

developer.

.LP

The XView video extension includes a global variable called

Xv_VideoStatus into which status codes will be placed after the

completion all operations. If a operation completed successfuly

(such as xv_create, xv_set) XView will normally return XV_OK to

indicate success.  If some error is detected, XView will return

XV_ERROR.  In this case, checking the value of Xv_VideoStatus

will result in additional information about the cause of the

problem.  Error codes which will be returned here are described

futher under the VIDEO_IN attributes which relate to them.

.LP

Another way errors are indicated is through parameter validty

checking.  Attributes which take values are subject to bounds

checks which will flag an error through the XView standard

error handling procedure if they occur.

.NH

ATTRIBUTES

.LP

The class VIDEO_IN offers many user controllable attributes which

influence the behavior of the video being displayed in the canvas

paint window.  These attributes are defined below.

.NH 2

\fBVIDEO_LIVE\fR

.LP

Setting this attribute to TRUE (the default) cause live video to

be shown in the canvas paint window.  Setting its value to FALSE

causes only the next available video frame to be digitized and

shown in the paint window.  The image will be cropped to fit the

size of the current canvas paint window.  The video data is

digitized and displayed according to the type of video input that

has been associated with the VIDEO_IN object specified by

VIDEO_INPUT_TYPE.

.IP

VIPER

.RS

.IP

Only one live window is allowed at a time.  If input type

VIDEO_YUV7 has been selected, the video is digitized in eight but

time-multiplexed YUV format.  If input type VIDEO_GRAY7 has been

specified, video is digitized as sven bit gray scale.  The default

input type is VIDEO_YUV7.

.RE

.IP

VIDEOVIEW:

.RS

.IP

Only one live window is allowed at a time.  The input type

specified by VIDEO_INPUT_TYPE is ignored, video is always

digitized in 24 bit RGB format (VIDEO_RGB24).

.RE

.IP

XVIDEO:

.RS

.IP

Up to two live windows are allowed at a time.  If input

type VIDEO_RGB24 has been specified with VIDEO_INPUT_TYPE, the

video is digitized in 24 bit RGB format.  If input type VIDEO_GRAY7

has been specified, video digitized as seven bit gray scale. If

VIDEO_GRAY8 has been specified, video is digitized as an eight bit

gray scale.

.RE

.NH 2

\fBVIDEO_SQUEEZE\fR

.LP

Setting this attribute to TRUE cause live video to be shown in the

canvas paint window.  However, the video image shown is created by

scaling the video data as bounded by the VIDEO_WIDTH and VIDEO_HEIGHT

values into the destination paint window using pixel sampling. The

format of the digitization follows the same rules as VIDEO_LIVE.

Additional size constraints are:

.IP

VIDEOVIEW:

.RS

.IP

The source width and height values are only selectable from

the following set: 640x480, 320x240, 160x120, 80x60, 40x30.

.RE

.NH 2

\fBVIDEO_SCALE\fR

.LP

NOT IMPLEMENTED

.NH 2

\fBVIDEO_NEW_FRAME\fR

.LP

VIDEO_NEW_FRAME is a write-only attribute which when set to TRUE,

causes the next available frame to be digitized and display in the

canvas paint window.  It is designed to be used in conjunction with

VIDEO_LIVE being set to FALSE.  The allows one to programmatically

select one frame at a time from the video source.

.NH 2

\fBVIDEO_FACTORY_RESET\fR

.LP

VIDEO_FACTORY_RESET is a read-only attribute which when accessed

causes the factory default values for video hue, saturation,

contrast, and brightness to be loaded into the board.  The value

returned is always TRUE. This attribute has no effect when used

during a call to xv_create.  Its proper usage would be:

.nf

video = (VideoIn) xv_create(frame,VIDEO_IN,NULL);

(void) xv_get(video,VIDEO_FACTORY_RESET);

.fi

\fBVIDEO_USER0\fR

.LP

VIDEO_USER0 is used to store and retrieve user defined values for

video hue, saturation, contrast, and brightness.  For example,

to define and save a set of video parameters, one could use the

following piece of code.

.nf

xv_set(video,

VIDEO_CONTRAST,		128,

VIDEO_BRIGHTNESS,	128,

VIDEO_SATURATION,	128,

VIDEO_USER0,			TRUE,

NULL);

.fi

The video hue value saved would be its current value in the board.

The actuals values are written into NVRAM (Non-Volitale RAM) so they

can be accessed at any latter session.

.nf

(void) xv_get(video,VIDEO_USER0);

.fi

This would reload and initialize the board to the previously saved

values.

.NH 2

\fBVIDEO_USER1\fR

.LP

This attribute performs exactly the same function as VIDEO_USER0,

but only exists on the XVideo product line.  It offers additional

storage for the user.

.NH 2

\fBVIDEO_SYNC\fR

.LP

VIDEO_SYNC is a read-only attribute which returns TRUE or FALSE

if the currently selected video input returns hardware detection

of a good sync source signal.

.NH 2

\fBVIDEO_INTERLACED\fR

.LP

VIDEO_INTERLACED is a read-only attribute which returns TRUE or

FALSE if the currently selected video input detects an interlaced

or non-interlaced video signal.

.NH 2

\fBVIDEO_NORMAL_ASPECT\fR

.LP

The attribute controls whether the video paint window is constrained

to only take on width and height values which reflect the normal

video signal aspect ratio of 4 to 3.  Setting this attribute to TRUE

causes the video paint window to enforce a width to height ratio of

4 to 3 at all times.  Setting the attribute to FALSE allows for the

video paint window to assume any unconstrained aspect ratio.

.NH 2

\fBVIDEO_TRACK_WIN\fR

.LP

This attribute specifies whether the video canvas should try to

track changes in the size of its parent frame.  The default (TRUE)

indicates that the video canvas will try to resize and fit the

parent frame.  Setting this value to FALSE will disable this

behavior.  This is desirable if you wish to create a fixed size

video canvas on a frame with other subwindows.

.NH 2

\fBVIDEO_X\fR

.LP

The starting point for the video digitization in the x direction is

controlled by this attribute.  It can assume any value between 0

(the default) and the maximum signal width derived from the signal

standard (VIDEO_NTSC, VIDEO_PAL, or VIDEO_SECAM).

.NH 2

\fBVIDEO_Y\fR

.LP

The starting point for the video digitization in the y direction is

controlled by this attribute.  It can assume any value between 0

(the default) and the maximum signal height derived from the signal

standard found on the input channel (VIDEO_NTSC, VIDEO_PAL, or

VIDEO_SECAM).

.NH 2

\fBVIDEO_WIDTH\fR

.LP

The number of video pels digitized in the x direction is controlled

by this attribute.  It can assume any value between 1 and the maximum

signal width (the default) derived from the signal standard found on

the input channel (VIDEO_NTSC, VIDEO_PAL, or VIDEO_SECAM).

.NH 2

\fBVIDEO_SIGNAL_WIDTH\fR

.LP

This is a read-only attribute which returns the width of the video

input signal as derived from the signal standard.

.NH 2

\fBVIDEO_HEIGHT\fR

.LP

The number of video lines digitized in the y direction is controlled

by this attribute.  It can assume any value between 1 and the maximum

signal height (number of lines) derived from the signal standard found

on the input channel (VIDEO_NTSC, VIDEO_PAL, or VIDEO_SECAM).  Its

default value is computed as the difference of the maximum number of

scan lines and the digitizable vertical interval in lines.

.NH 2

\fBVIDEO_SIGNAL_HEIGHT\fR

.LP

This is a read-only attribute which returns the height of the video

input signal as derived from the signal standard.

.NH 2

\fBVIDEO_CHANNEL\fR

.LP

The video being digitized into the canvas paint window is accessed

through an input channel from an external source.  Valid channels

are VIDEO_IN_ONE and VIDEO_IN_TWO.  If the requested channel does

not exists (such as channel two on a VideoView board) then Xv_VideoStatus

is set to VIDEO_NO_MORE_CHANNELS.  If channel exists, but is already

is inuse, then Xv_VideoStatus is set to VIDEO_CHANNEL_IN_USE.

.NH 2

\fBVIDEO_STANDARD\fR

.LP

The VIDEO_STANDARD values recognized are VIDEO_NTSC, VIDEO_PAL, and

VIDEO_SECAM.  This parameter servers as only a hint when the channel

is openned, the actual video standard detected will override this

attribute.  If the active channel does not support this video standard,

then Xv_VideoStatus is set to VIDEO_BAD_STANDARD.

.NH 2

\fBVIDEO_FORMAT\fR

.LP

This attribute is used to select the desired video input format. Valid

formats supported are VIDEO_COMPOSITE, VIDEO_SUPER_VHS, VIDEO_YUV, and

VIDEO_RGB.  Often, each format corresponds to a different physical

connector on the video board.  If the active channel does not support

this format, then Xv_VideoStatus is set VIDEO_BAD_FORMAT.

.NH 2

\fBVIDEO_INPUT_TYPE\fR

.LP

The attribute controls the format of the video data as it is digitized

off the video input channel.  Supported input types (all not available

on all boards) are VIDEO_YUV7, VIDEO_RGB24, VIDEO_RGB8, VIDEO_GRAY7,

and VIDEO_GRAY8.  If the underlying video hardware can not generate

this data format, then Xv_VideoStatus is set to VIDEO_BAD_INPUT_TYPE.

.NH 2

\fBVIDEO_SATURATION\fR

.LP

This attribute accesses the saturation level associated with the video

input channel.  It can assume any value from 0 to 255.

.NH 2

\fBVIDEO_HUE\fR

.LP

This attribute accesses the hue associated with the video

input channel.  It can assume any value from 0 to 255.

.NH 2

\fBVIDEO_CONTRAST\fR

.LP

This attribute accesses the contrast level associated with the video

input channel.  It can assume any value from 0 to 255.

.NH 2

\fBVIDEO_BRIGHTNESS\fR

.LP

This attribute accesses the brightness level associated with the video

input channel.  It can assume any value from 0 to 255.

.NH 2

\fBVIDEO_GRAPHICS\fR

.LP

NOT IMPLEMENTED

.NH 2

\fBVIDEO_HARDWARE\fR

.LP

This is a read-only field which returns the hardware type installed

in the workstation and whose values are defined in Video.h.

.NH 2

\fBVIDEO_PIXRECT\fR

.LP

This attribute can be used to access the video data in the form of a

memory pixrect.  It can be used in conjunction with xv_get to retrieve

a copy of the video data as visible on the display stored as a memory

pixrect.  It can be used to load the video frame buffer with an existing

memory pixrect via xv_set.  VIDEO_LIVE must be set to FALSE when using

this attribute.

.NH 2

\fBVIDEO_XIMAGE\fR

.LP

This attribute can be used to access the video data in the form of a

XImage data structure.  It can be used in conjunction with xv_get to

retrieve a copy of the video data as visible on the display stored in

memory as a XImage structure.  It can be used to load the video frame

buffer with an existing XImage structure via xv_set.  VIDEO_LIVE must

be set to FALSE when using this attribute.

.NH 2

\fBVIDEO_EVENT_PROC\fR

.LP

This attribute can be used to specify an window event procedure for

the video canvas which parents the video input object.  The video

event proc can't be specified until the VideoIn object has been

created.  This is because the event function is attached to the

video paint window, which doesn't exist until the initialization

call has returned.  Thus a two step process is required.

.NH 2

\fBVIDEO_GC\fR

.LP

This attribute can be accessed to get the X11 graphics context (GC)

from the video_in object. Note that this parameter is read-only.

.NH 2

\fBVIDEO_SAVE_STILL\fR

.LP

This attribute is a boolean which controls whether a still video

image is cached in the video_in object. By default, this attribute

is set to FALSE, which means that whenever a repaint operation

occurs in the video_in canvas, the current video frame from the

source is used to repaint the screen. When set to TRUE, whenever

the video window is STILLED, the current frame is cached as the

source for repainting the video window when it is damaged.

.nf

video = (VideoIn) xv_create(frame,VIDEO_IN,NULL);

(void) xv_set(video,VIDEO_EVENT_PROC,event_handler,NULL);

.fi

.LP

The class VIDEO_OUT offers many user controllable attributes which

influence the behavior of the video being output from the screen

and through the board.  These attributes are defined below.

.NH 2

\fBVIDEO_OUTPUT\fR

.LP

Setting this attribute to TRUE causes the currently defined video

output region to be output from the framebuffer through the output

channel.  The default value of the parameter is FALSE.

.NH 2

\fBVIDEO_OUT_SCALE\fR

.LP

Setting this attribute to TRUE (the default) causes the contents of

the currently defined video output region to be scaled up to 640 by

482 if it is smaller.  Any region defined larger than this is cropped

to 640 by 480.  If the value is set to FALSE, then any region smaller

than 640 by 480 is output as a subset of a 640 by 480 video field.

.NH 2

\fBVIDEO_OUT_ASPECT\fR

.LP

Setting this attribute to TRUE (the default) causes the contents of

the currently defined video output region to be scaled up such that

a 4 to 3 aspect ratio is always maintained for video output.

.NH 2

\fBVIDEO_OUT_X\fR

.LP

.NH 2

This attribute is used to specify the upper left corner of the

bounding box used to defined the active video output region in

the frame buffer.

\fBVIDEO_OUT_Y\fR

.LP

.NH 2

This attribute is used to specify the upper top corner of the

bounding box used to defined the active video output region in

the frame buffer.

\fBVIDEO_OUT_WIDTH\fR

.LP

.NH 2

This attribute is used to specify the width of the bounding box

used to defined the active video output region in the frame buffer.

\fBVIDEO_OUT_HEIGHT\fR

.LP

.NH 2

This attribute is used to specify the height of the bounding box

used to defined the active video output region in the frame buffer.

\fBVIDEO_OUT_STANDARD\fR

.LP

The VIDEO_STANDARD values recognized are VIDEO_NTSC, VIDEO_PAL, and

VIDEO_SECAM.  This parameter servers as only a hint when the channel

is openned, the actual video standard detected will override this

attribute.  If the active channel does not support this video standard,

then Xv_VideoStatus is set to VIDEO_BAD_STANDARD.

.NH 2

\fBVIDEO_OUT_FORMAT\fR

.LP

This attribute is used to select the desired video output format. Valid

formats supported are VIDEO_COMPOSITE, VIDEO_SUPER_VHS, VIDEO_YUV, and

VIDEO_RGB.  Often, each format corresponds to a different physical

connector on the video board.  If the active channel does not support

this format, then Xv_VideoStatus is set VIDEO_BAD_FORMAT.

.NH 2

\fBVIDEO_OUTPUT_TYPE\fR

.LP

The attribute controls the format of the video data as it is transferred

down the video output channel.  Supported output types (all not available

on all boards) are VIDEO_YUV7, VIDEO_RGB24, VIDEO_RGB8, VIDEO_GRAY7,

and VIDEO_GRAY8.  If the underlying video hardware can not generate

this data format, then Xv_VideoStatus is set to VIDEO_BAD_OUTPUT_TYPE.

.NH 2

\fBVIDEO_OUT_CHANNEL\fR

.LP

.NH 2

The video being transfered from the framebuffer is sent through an

output channel to the external destination.  The only valid channel

is VIDEO_OUT_ONE.  If an output channel does not exist (such as on

a Viper board) then Xv_VideoStatus is set to VIDEO_NO_MORE_CHANNELS.

If channel exists, but is already is inuse, then Xv_VideoStatus is

set to VIDEO_CHANNEL_IN_USE.

\fBVIDEO_OUT_HARDWARE\fR

.LP

This is a read-only field which returns the hardware type installed

in the workstation and whose values are defined in Video.h.