Name                Class              RepType         Default Value   Access

----                -----              -------         -------------   ------

XtNaspect	     XtCAspect		Boolean		False		CSG

Directs the configuration management to keep the aspect ratio at 4 to 3.

XtNzoomWidget	     XtCzoomWidget	Widget		NULL		CSG

Widget that's upper left hand corner will be the origen of the zoom, else

the video widget will be the origen.

XtNinputType 	     XtCInputType	int 		from board	SG

The standard parameter of XPlxInputSelect. (PLX_NTSC, PLX_PAL, PLX_SECAM)

XtNinputFormat      XtCInputFormat	int 		from board	SG

The format parameter of XPlxInputSelect. (PLX_COMP, PLX_YC, PLX_YUV,

PLX_RGB).

XtNwindowType	     XtCWindowType 	int 		from board 	SG

The type parameter of XPlxInputSelect. (PLX_YUV7, PLX_RGB24, PLX_RGB8,

PLX_GRAY5, PLX_GRAY7, PLX_GRAY8).

XtNoutputType 	     XtCOutputType 	int 		from board	SG

The type parameter of XPlxOutputSelect. (see XtNwindowType)

XtNoutputFormat     XtCOutputFormat	int 		from board	SG

The format parameter of XPlxOutputSelect. (see XtNinputFormat)

XtNoutputChannel    XtCOutputChannel	int 		PLX_IO_OFF	CSG

The channel parameter of XPlxOutputSelect. (see XtNchannel)

XtNresizeParent     XtCResizeParent	Boolean		False		CSG

Directs the configuration management to try and resize it's parent.

(see configruation management below)

XtNconfigWidget     XtCConfigWidget	Widget		NULL		CG

The widget that should be moved to satisfy any allignment restrictions or

placement notification requirements. (see configuration management below)

XtNplx_IO	     XtCPlx_IO		plx_IO		from board	G

The plx_IO structure pointer returned by XPlxQueryConfig can be obtained

any time after creation.

XtNwindowGravity    XtCWindowGravity	int		NorthWestGravity CSG

The Gravity to use for placement of the video window within it's parent

when the parent is not resized. (see configuration management below)

Configuration Management:

The 1280, Viper, and VideoView displays all require some configuration

management. The 1280 and Viper boards have allignment restrictions of

x % 16 == 0, with a width restriction of w % 16 == 0. The VideoView,

since it is client based and not part of the server, needs to be

notified of when it's origen or size has changed. The video widget will

take care of this with a little help from the application developer.

Since (new paragraph here i would think)

no notification of movement of a child is given if the parent has moved,

an event handler needs to be attached to the top level widget ancester of

the video widget.  Therefore, either the widget returned from XtInitialize,

or another top level widget such as an ApplicationShell widget should be

passed in at creation time via the XtNconfigWidget argument. Restrictions

on size, whether imposed by the aspect argument or necesitated by the

hardware are handled as follows. If the aspect argument is true the video

widget is resized such that width to height is 4 to 3 based on whichever,

width or height, results in a smaller window. On the 1280 and Viper

displays the widget is allways resized down to accomodate the width

restriction mentioned above. On VideoView displays, if the squeeze argument

is true, and the aspect argument is true, the widget is resized to the next

size that it is possible to squeeze to. If the resizeparent argument is true

and the widget was resized then an attempt is made to resize the parent.

This will only work if the parent is a top level widget. If the parent

cannot, for whatever reason, be resized to fit the video widget's

size then the windowgravity argument is used to place the video widget

within its parent. In order for this to work properly, the video widget

should be the only child of it's parent.